# ADE_Analytical_Multimetal.py — Input Reference

W.M. Kanchana D. Wijeratna  |  Hydro-Environment System Laboratory, Tohoku University

# 1. Input overview

| Input | Type | Purpose |
| --- | --- | --- |
| clone_map | PCRaster .map file | Defines grid dimensions, projection, NoData mask |
| ldd_map | PCRaster .map file | Local drain direction — encodes river network topology |
| pollutant_map | PCRaster .map file | Base spatial grid for source cell locations |
| flow_map (time series) | PCRaster .map files | Discharge Q(x,t) at every cell, one file per timestep |
| source_signatures.csv | CSV file | Source locations, total masses, and per-metal fractions |
| CONFIG parameters | Python dict (in script) | Numerical settings, ADE physics, file paths |

# 2. PCRaster map files

## 2.1  clone_map

Defines the spatial reference for the entire simulation: grid dimensions (rows × columns), cell size, coordinate system, and NoData pattern. Every other PCRaster map must be cloned from this file.

CONFIG key:  "clone_map"

Example:     pcr_dem.map

Note: This is the first file loaded. If it is missing or corrupt, the model crashes immediately with a PCRaster clone error.

## 2.2  ldd_map

Local drain direction map. Each cell contains an integer 1–9 indicating which of its 8 neighbours it drains into (5 = pit / outlet). PCRaster uses this to compute upstream() accumulation and route mass through the river network, including all confluences.

CONFIG key:  "ldd_map"

Example:     WGS_LDD.map

Note: Must be a sound LDD (no unsound cells). Generate with pcr.lddcreate() from your DEM or import a pre-computed LDD.

## 2.3  pollutant_map

Base raster used only to initialise the spatial structure (array shape, NoData pattern) before the signature CSV overrides specific cells. Its numeric values are overwritten by the CSV. It does NOT need to contain correct mass values — it just needs to have the right grid.

CONFIG key:  "pollutant_map"

Example:     pollution_source_WGS_200points.map

## 2.4  flow_map (discharge time series)

A sequence of PCRaster scalar maps giving discharge Q [m³/s] at every channel cell for each timestep. The model reads one file per timestep using the PCRaster dynamic framework naming convention:

| Timestep | Filename pattern | Example |
| --- | --- | --- |
| 1 | Q0000000.001 | Q0000000.001 |
| 999 | Q0000000.999 | Q0000000.999 |
| 1000 | Q0000001.001 | Q0000001.001 |
| 2000 | Q0000002.001 | Q0000002.001 |

Formula: block = (step-1) // 999,  slice = (step-1) % 999 + 1

CONFIG key:  "flow_map"  (prefix string only, e.g. ".../Discharge/Q")

Note: The model appends the block and slice numbers automatically. Provide only the path prefix without any numeric suffix.

# 3. source_signatures.csv  (the key new input)

This is the central new input that did not exist in the original FVM model. It replaces the old Replacement_pollution_source CSV and extends it to carry per-metal chemical signatures for every source cell.

## 3.1  Column definitions

| Column | Type | Units / format | Description |
| --- | --- | --- | --- |
| source_id | string | any unique label | Identifier for this source (e.g. SRC_01, factory_A). Used in log output only. |
| row | integer | 1-based PCRaster row | Row of the source cell in the raster grid. Row 1 is the top row. |
| col | integer | 1-based PCRaster column | Column of the source cell. Column 1 is the leftmost column. |
| total_mass | float | mg | Total pollutant mass released by this source during the release window. |
| Cd | float | fraction [0–1] | Fraction of total_mass that is Cadmium. All metal fractions in a row must sum to 1.0. |
| Cr | float | fraction [0–1] | Fraction that is Chromium. |
| Pb | float | fraction [0–1] | Fraction that is Lead. |
| Cu | float | fraction [0–1] | Fraction that is Copper. |
| Zn | float | fraction [0–1] | Fraction that is Zinc. Add or remove metal columns freely — the model reads them dynamically. |

## 3.2  Example CSV content

| source_id | row | col | total_mass | Cd | Cr | Pb | Cu | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SRC_01 | 45 | 12 | 500.0 | 0.16 | 0.21 | 0.18 | 0.11 | 0.34 |
| SRC_02 | 60 | 30 | 800.0 | 0.16 | 0.34 | 0.25 | 0.24 | 0.01 |
| SRC_03 | 80 | 55 | 300.0 | 0.32 | 0.24 | 0.01 | 0.26 | 0.17 |
| SRC_04 | 100 | 70 | 650.0 | 0.23 | 0.13 | 0.29 | 0.18 | 0.17 |

## 3.3  Validation rules

- Each row's metal fractions must sum to 1.0  (tolerance ±0.001). The loader will warn and auto-normalise if they do not.

- row and col are 1-based (PCRaster convention). row=1 is the top row of the grid.

- Any source cell outside the grid bounds is skipped with a warning.

- Metal column names can be anything (Al, Fe, Ni, …). The set of metals is read from the CSV header — no code change required.

- Multiple rows with the same (row, col) are allowed and their masses are additive per metal.

## 3.4  How total_mass relates to emission rate E

Inside the model, total_mass is converted to an emission rate per unit length:

E_metal [mg/m] = total_mass [mg] × fraction[metal] / deltaX [m]

This is then applied only during the release window [release_start, release_end]. The deltaX value comes from CONFIG.

# 4. CONFIG parameters (set in the Python script)

## 4.1  File paths

| Parameter | Type | Description |
| --- | --- | --- |
| output_dir | string path | Directory where all output maps and logs are written. Created automatically if missing. |
| clone_map | string path | Path to the PCRaster clone map (.map file). |
| ldd_map | string path | Path to the local drain direction map (.map file). |
| pollutant_map | string path | Path to the base pollutant raster (used for grid structure only). |
| flow_map | string path prefix | Prefix for discharge time-series files. Do not include the numeric suffix. |
| signature_csv | string path | Path to source_signatures.csv (Section 3). |

## 4.2  Numerical / temporal parameters

| Parameter | Default | Units | Description |
| --- | --- | --- | --- |
| deltaT | 1 | seconds | Timestep size. Must satisfy CFL condition: u·deltaT/deltaX ≤ 1. |
| deltaX | 100 | metres | Grid cell size (spatial step). Determines spatial resolution. |
| velocity | 1.8 | m/s | Mean flow velocity. Used as fallback when Q maps are unavailable and to compute cross-section A = Q/velocity. |
| release_start | 1 | timestep | First timestep at which sources emit pollutants. |
| release_end | 2 | timestep | Last timestep of pollutant release. After this, E = 0. |
| nrOfTimeSteps | 2000 | timesteps | Total number of simulation steps. Simulation duration = nrOfTimeSteps × deltaT. |
| ocean_cell | (278, 18) | (row, col) | 1-based grid coordinates of the outlet/ocean cell used to track cumulative mass leaving the domain. |

## 4.3  Analytical ADE physics parameters  (new vs FVM version)

These parameters have no equivalent in the original upwind FVM model. They control the physical dispersion and decay of pollutants.

| Parameter | Default | Units | Description |
| --- | --- | --- | --- |
| Ex | 10.0 | m²/s | Longitudinal dispersion coefficient. Controls physical plume spreading. Set to 0 for pure advection. Typical rivers: 1–100 m²/s. The FVM upwind scheme had implicit numerical diffusion Ex_num = u·deltaX/2 = 90 m²/s — setting Ex < 90 gives sharper, more realistic plumes. |
| K | 0.0 | 1/s | First-order decay/retardation coefficient. Set to 0 for conservative heavy metals (Cd, Cr, Pb, Cu, Zn). Set K > 0 for reactive tracers. |
| n_max | 100 | integer | Number of eigenfunction terms in the series solution. Higher values improve accuracy for sharp concentration fronts. 50–200 is sufficient for smooth plumes; 500+ for very sharp pulses. Computational cost scales linearly with n_max. |
| segment_length | 5000.0 | metres | Effective 1-D domain length L for the eigenfunction series boundary conditions. Set to the approximate length of the longest channel segment between two confluences. Does not need to match the total basin size. |

Note: Numerical diffusion of the old upwind scheme: Ex_numerical = u × deltaX / 2 = 1.8 × 100 / 2 = 90 m²/s. Any Ex < 90 m²/s will produce sharper plumes than the FVM model.

## 4.4  Reporting options

| Parameter | Default | Description |
| --- | --- | --- |
| report_concentration | True | If True, saves concentration C [mg/m³] maps alongside mass M [mg/m] maps. Doubles output storage. Set False to save only M maps. |

# 5. Output files produced

| File / pattern | Description |
| --- | --- |
| <output_dir>/<METAL>/M_<METAL>XXXXXXX.XXX | Mass per unit length map [mg/m] for each metal, each timestep. One subdirectory per metal (Cd/, Cr/, etc.). |
| <output_dir>/<METAL>/C_<METAL>XXXXXXX.XXX | Concentration map [mg/m³], only if report_concentration=True. |
| <output_dir>/mass_balance_log.csv | Timestep × metal mass balance log: columns timestep, metal, Theoretical_Total, Model_Total, Error, ErrorPct. |
| <output_dir>/source_metal_maps/E_nominal_<METAL> | Diagnostic: initial emission rate map for each metal [mg/m]. Useful for verifying that the CSV was read correctly. |

# 6. Run modes

| Command | Mode | What it does |
| --- | --- | --- |
| python ADE_Analytical_Multimetal.py | basin (default) | Full PCRaster basin simulation with analytical dispersion. Requires all map files and signature CSV. |
| python ADE_Analytical_Multimetal.py --mode validate | validate | 1-D comparison of FVM upwind vs analytical solution. No PCRaster maps needed. Saves a PNG plot. |
| python ADE_Analytical_Multimetal.py --mode 1d | 1d | Standalone multi-metal analytical model for a single channel. Uses hardcoded example data in the script. No PCRaster maps needed. |

# 7. Pre-run checklist

- clone_map path is correct and the file exists

- ldd_map covers the same extent and resolution as clone_map

- flow_map prefix points to the correct directory; Q0000000.001 exists

- signature_csv exists; open it and verify fraction columns sum to 1.0 per row

- release_start and release_end are within [1, nrOfTimeSteps]

- CFL condition satisfied: velocity × deltaT / deltaX ≤ 1  (default: 1.8×1/100 = 0.018 ✓)

- ocean_cell (row, col) falls on a valid channel cell in the grid

- output_dir has write permission (created automatically if missing)